Practical 4

Controlling LED using Telegram App

Steps: -

- Click on Raspberry Icon > Programming > Thonny Python IDE

- On the Right Side Click on the Switch to regular mode > Close and reopen Thonny

- From the upper tab click on Tools > Manage Packages > Search telepot and Install

OR Use Below Steps

In the raspberry pi PC open Terminal and type

pip3 install telepot

If not getting install or connection error go to File explorer > /etc > dhcpcd.conf file

Edit the IP address earlier it was – 192.168.0.1   Change to 192.168.2.1

| LED Terminal         | Pin Number        | GPIO Number |

|---------------------|------------------|-------------|

| Red Positive        | Pin 15 (LED 1)   | GPIO 22     |

| Blue Positive     | Pin 16 (LED 2)   | GPIO 23     |

| Ground for LED 1    | Pin 9            | GND         |

| Ground for LED 2    | Pin 6            | GND         |

- Install Telegram app in phone.

- Search BotFather and Add

- Click on Open > create a bot > name the bot and username

- Copy the bot link for the further coding.

Code: -

import telepot

from telepot.loop import MessageLoop

from datetime import datetime

import RPi.GPIO as GPIO

from time import sleep

red = 22

yellow = 23

now = datetime.now()

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(red, GPIO.OUT)

GPIO.output(red, 0)

GPIO.setup(yellow, GPIO.OUT)

GPIO.output(yellow, 0)

def action(msg):

chat_id = msg["chat"]["id"]

command = msg["text"]

print("Recieved: " + command)

if "on" in command.casefold():

message = "on "

if "red" in command.casefold():

message = message + "red"

GPIO.output(red, 1)

elif "yellow" in command.casefold():

message = message + " yellow"

GPIO.output(yellow, 1)

message = message + " light(s)"

telegram_bot.sendMessage(chat_id, message)

if "off" in command.casefold():

message = "off "

if "red" in command.casefold():

message = message + " red"

GPIO.output(red, 0)

elif "yellow" in command.casefold():

message = message + " yellow"

GPIO.output(yellow, 0)

message = message + " light(s)"

telegram_bot.sendMessage(chat_id, message)

telegram_bot = telepot.Bot("8293210490:AAGfJLjRkgsJTE3DbbAuY_pHtWl1sOMSrLA")

print(telegram_bot.getMe())

MessageLoop(telegram_bot, action).run_as_thread()

try:

while True:

sleep(10)

finally:

GPIO.output(red, 0)

GPIO.output(yellow, 0)

GPIO.cleanup()

- In Telegram Search BotFather > Open the chats

- Message /mybots

- Then choose the mybots and click on the @username

- Also Run the python code

- Message -  /start

On red

Output: -