Practical 7

Using Temperature & Humidity Sensor

with Raspberry Pi

To interface a DHT11 Temperature & Humidity Sensor with Raspberry Pi 3 and display the temperature and humidity values using Python.

Theory about Temperature & Humidity sensor DHT11

Components Required

- Raspberry Pi 3

- DHT11 Temperature & Humidity Sensor (module preferred)

- Jumper wires (Male–Female)

- Breadboard (optional)

- Micro-SD card with Raspberry Pi OS

- Internet connection (for library installation)

Pin connections

| DHT11 Pins | Raspberry Pi Pins | GPIO Numbering |
| --- | --- | --- |
| VCC (+) | Pin 1 | 3.3V |
| Data (Out) | Pin 7 | GPIO4 |
| GND (-) | Pin 6 | Ground |

| DHT11 Pins  | Raspberry Pi Pins | GPIO Numbering |

|-------------|-------------------|----------------|

| VCC (+)     | Pin 1             | 3.3V           |

| Data (Out)  | Pin 7             | GPIO 4         |

| GND (-)     | Pin 6             | Ground         |

Install Required Libraries

Directly install the libray using Thonny > Manage Packages > Search for Adafruit_DHT > Install

- sudo apt install python3-pip  (if already installed need not install again)

- pip3 install Adafruit_DHT

Digital Temperature and Humidity Sensor

Version :- DHT11

Code: -

'''

To interface a DHT11 Temperature & Humidity Sensor with Raspberry Pi 3

and display the temperature and humidity values using Python.

'''

import Adafruit_DHT

import time

# Sensor type

sensor = Adafruit_DHT.DHT11

# GPIO pin where DATA is connected

gpio_pin = 4

while True:

humidity, temperature = Adafruit_DHT.read(sensor, gpio_pin)

if humidity is not None and temperature is not None:

print("Temperature = {:.1f}°C".format(temperature))

print("Humidity = {:.1f}%".format(humidity))

print("------------------------------")

else:

print("Failed to retrieve data from sensor")

time.sleep(2)

Output: -